Projector Firmware Upgrade SOP

Requirements

Software

- DLP Composer Lite (.exe)

- Projector Firmware image file (*.img)

- Library file (FlashDeviceParameters.txt)

Hardware

- Projector and Projector power cord

- USB cable (USB-A to Micro USB)

- PC or Laptop (Windows XP 32-bit or greater)

Install DLP Composer Lite Software

- Install DLP Composer Lite (version maybe newer)

- Follow related process, click “Next” until “Finish”.

- Copy “FlashDeviceParameters.txt” to the path—C:\Program Files(x86)\DLP Composer Lite Folder

Loading Upgrade Firmware

- Connect a USB-A to Micro USB cable between PC USB port and projector’s service port.

- Press and hold Menu and Power while plugging in the power cable into the projector then release the buttons as soon as you see TEMP/LAMP/POWER red LED lights to enter programming mode.

- Open “DLP Composer(TM) Lite” program from Windows in start menu

- Open menu “Edit > Preferences” select “Communications, then USB”, and then click “OK”.

- Select “Flash Loader”, click “Browse…” choose firmware .img file. Example, “DDP4422.img”.

- In Flash Loader choose “Complete Image Download (slowest: downloads entire image)” and enable “Skip Boot Loader Area: 128KB”.

- Click “Reset Bus”, and then click “Start Download” to start download firmware.

- Once the download has completed, remove AC power cord and the USB cable. After the LED lights goes out, plug in the power cord, turn it on, and confirm the firmware has upgraded in the projector menu.